4. Shaft Stress Validation (Analytical vs ANSYS)

4.1 Given Parameters

From the mechanical design stage:

| Parameter | Symbol | Value |
| --- | --- | --- |
| Shaft diameter | d | 12 mm |
| Shaft radius | r | 6 mm |
| Applied torque | T | 3 N·m |

Convert to SI units

d=0.012 m
r=0.006 m
T=3 N⋅m

Source:

4.2 Polar Moment of Inertia

For a solid circular shaft, the polar moment of inertia is

J=πd432

Substituting

J=π(0.012)432
J=2.04×10-9 m4

4.3 Maximum Shear Stress

The torsional shear stress in a solid shaft is

τmax=TrJ

Substituting values

τmax=3×0.0062.04×10-9
τmax=8.82×106 Pa
τmax=8.82 MPa

Thus, the maximum torsional shear stress in the shaft is

τmax≈8.8 MPa

4.4 von Mises Equivalent Stress

For pure torsion, the equivalent von Mises stress is

σvm=3 τ

Substituting

σvm=1.732×8.82
σvm=15.3 MPa

Thus, the equivalent von Mises stress in the shaft is

σvm≈15.3 MPa

4.5 ANSYS Simulation Result

Finite element analysis of the shaft in ANSYS produced a maximum stress of

σANSYS=1.6009×107 Pa
σANSYS=16.0 MPa

Source:

4.6 Error Percentage

The difference between analytical and numerical results is calculated as

Error=∣σANSYS-σanalytical∣σANSYS×100

Substituting

Error=∣16.0-15.3∣16.0×100
Error=4.3%

4.7 Validation Conclusion

| Method | Stress |
| --- | --- |
| Analytical calculation | 15.3 MPa |
| ANSYS simulation | 16.0 MPa |

The difference between the analytical solution and the finite element simulation is approximately 4.3%, which indicates excellent agreement. This confirms that:

- the applied torque is correctly modeled

- the boundary conditions in ANSYS are appropriate

- the analytical mechanical design calculations are valid.